# Current Wiring Schematic

This diagram shows how your control module (ESP32C3 + INA219 + Relay + Protections) is currently wired to drive and monitor a 24 VDC ballast.

+24VDC PSU ──┐

│

│  PTC Fuse (RUEF200)

│    [PTC]

│      │

└──► VIN+ (INA219) ──┐

│  (internal shunt)

│

VIN– (INA219) ──► COM (Relay)

│

│ NO (Relay)

│

+24V Ballast

+24VDC PSU ──┐

│

│  DC-DC Buck/LDO Regulator

│    – In: +24V post-fuse

│    – Out: +3.3V (3V3 Bus)

│

–24VDC PSU ────────────────────────────────┐

│

▼

Common GND /

Logic Ground

(ESP32, INA219, TVS, capacitor)

I2C:

ESP32 D4 (SDA) ──────────────────────────► INA219 SDA

ESP32 D5 (SCL) ──────────────────────────► INA219 SCL

Relay control:

ESP32 D6 ────────────────────────────────► Relay IN (active LOW)

3V3 Bus:

3V3 ─────────────────────────────────────► ESP32C3 3V3

3V3 ─────────────────────────────────────► INA219 VCC

3V3 ─────────────────────────────────────► Relay VCC

## Connector and Component Description

- 24 VDC PSU:

  - +24 V (black) → PTC fuse → VIN+ of INA219

  - –24 V (white/blue) → common GND

- PTC Fuse (RUEF200): series overcurrent protection on +24 V

- TVS Diode (30 V) (https://www.aliexpress.com/item/32878977126.html) and 330 µF/35 V Capacitor  (https://www.aliexpress.com/item/1005006186858370.html): placed in parallel between +24 V post-fuse and common GND

- DC-DC Buck Converter / LDO Regulator (https://www.aliexpress.com/item/1005001629723875.html):

  - Input: +24 V after PTC fuse

  - Output: +3.3 V (3V3 Bus) to power ESP32C3 and INA219

  - Must supply ≥500 mA with low ripple

- INA219 (https://www.aliexpress.com/item/1005006109458834.html) (high-side current sense):

  - VIN+ → +24 V after PTC fuse

  - VIN– → COM of relay

  - VCC → +3.3 V

  - GND → common GND

  - SDA → GPIO4 (D4)

  - SCL → GPIO5 (D5)

- 3 V Relay Module (https://www.aliexpress.com/item/1005006334524742.html):

  - COM → VIN– of INA219 (relay input)

  - NO → +24 V to ballast

  - VCC → +3.3 V

  - GND → common GND

  - IN → GPIO6 (D6), active LOW

- ESP32C3 XIAO (https://www.aliexpress.com/item/1005006353129383.html):

  - 3.3 V → powers relay VCC and INA219 VCC

  - GND → common GND

  - D4 (GPIO4) → SDA of INA219

  - D5 (GPIO5) → SCL of INA219

  - D6 (GPIO6) → IN of relay

- Ballast (T5 UV Lamp):

  - +24 V → from NO of relay

  - –24 V → common GND (not via INA219)

  - Green/Chassis → chassis ground (not tied to logic GND)

NOTE:

+24VDC and -24VDC are drawn from the existing HVAC PCB rather than directly from the PSU.