The entire build must be done in a Linux machine (virtual or real) connected to our network or our VPN. If using a VM, make sure a 2TB ‘real’ HDD or SSD is forwarded to it only to build the system. Virtual HHDs will work really slow and it will definitely take over 1 day to build the entire distribution from scratch. Although Microsoft WSL hasn’t been tested, it should work faster but it would be really hard to have a visual environment (not needed to build yocto).

Requirements:

sudo apt install chrpath

sudo apt install diffstat

sudo apt-get install graphviz

sudo apt-get install -y gettext autopoint

sudo dpkg-reconfigure dash

Answer NO

----------------------------------------------------------------------------------------------------------------------------

For any doubts we followed: https://variwiki.com/index.php?title=Yocto_Build_Release&release=am62-yocto-kirkstone-6.1.46_09.01.00.08-v1.0

We still use the same base build system (without direct modifications) and customizing it through ‘meta-trismed’ layer.

----------------------------------------------------------------------------------------------------------------------------

Our Yocto build system has 4 repositories hosted in out git server, but only one of them is manually cloned:

- Git clone http://192.168.10.251/Bonobo.Git.Server/yocto-trismed.git

This is the only one that need to be manually cloned in to start building our software.

After cloning is done this command must be ran:

./FetchAll.sh ronny superStrongPassword

ronny: user with access to the repository to allow the script to clone ‘meta-trismed’ layer

superStrongPassword: my account password

Note: Miss Lee password is also valid for this. New users can be created and grant access to our repositories if needed.

Note: It is recommended to review the content of ‘./FetchAll.sh’ to understand what is being done.

Important: More details are available in ‘README.md’ after cloning this repository.

- http://192.168.10.251/Bonobo.Git.Server/meta-trismed.git

This is our company layer, it contains new recipes that where not available in the original Yocto distribution, and some fixed and tweaking required on the original Yocto system (done using ‘.bbappend’). It has the ‘image recipes’ that we build for Cardipia.

Note: This is the repository that need to be modified.

Read ‘meta-trismed/README.md’ for more details about what is being done in this layer.