# Wearable Hardware Engineer / Product Designer

## Smartband Development for Physiological Data Capture

We are looking for an experienced wearable hardware engineer or product designer to develop a low-cost smartband/wrist wearable prototype that captures raw physiological sensor data and transmits it wirelessly to our platform in real time.

The goal is to create a manufacturable prototype suitable for small-batch production via PCBWay and future scaling into clinical and commercial pilots.

## Project Overview

The device will function as a lightweight, screenless smartband focused on physiological signal capture for mental health and exposure therapy applications.

The wearable must:

- Capture raw PPG (photoplethysmography) data

- Capture raw GSR/EDA (galvanic skin response) data

- Stream data wirelessly to a mobile phone or gateway

- Be low-cost and manufacturable

- Support future expansion for additional sensors

This is not a consumer smartwatch. Simplicity, reliability, battery life, and clean raw data access are more important than polished UX features.

## Initial Sensor Components

### PPG Sensor

Using:

- SparkFun Pulse Sensor SEN-11574

Requirements:

- Access to raw waveform data

- Adjustable sampling rate

- Motion-noise mitigation considerations

- Continuous streaming support

### GSR / EDA Sensor

Using:

- ProtoCentral TinyGSR Sensor

Requirements:

- Raw conductance/resistance data access

- Stable skin contact design

- Low-noise signal acquisition

- Continuous streaming support

## Scope of Work

### Hardware Design

Design a wearable smartband including:

- PCB schematic and layout

- Microcontroller selection

- BLE wireless transmission

- Power management system

- Battery charging circuitry

- Sensor integration

- Compact wearable form factor

- Manufacturable enclosure design

### Firmware Development

Develop firmware capable of:

- Sampling raw PPG + GSR data

- BLE streaming to mobile device

- Timestamp synchronization

- Adjustable sampling rates

- Basic buffering and packet handling

- OTA or USB firmware update capability

### Industrial Design

Design considerations:

- Comfortable wrist-worn form factor using common straps

- No display required

- Rechargeable via USB-C

- Lightweight and durable

- Good sensor-to-skin contact

- Sweat and movement tolerance

- 3D printable enclosure

### Manufacturing Readiness

Deliverables should support manufacturing through PCBWay, including:

- Gerber files

- BOM

- Assembly files

- PCB layout files

- Firmware source code

- Enclosure CAD/STL files

- Manufacturing documentation

- Recommended production approach

## Suggested Technical Architecture

Open to recommendations, but expected architecture may include:

- Nordic nRF52 or ESP32 MCU

- BLE 5.0+

- LiPo battery

- Battery management IC

- I2C/SPI sensor integration

- Raw data transmission protocol

## Key Requirements

### Critical

- Full access to raw physiological data

- No locked ecosystem

- BLE streaming capability

- Low-cost BOM

- Reliable continuous data capture

- Manufacturable prototype

### Nice to Have

- IMU/accelerometer support

- Offline buffering

- Open-source firmware

- Waterproofing considerations

- Future expansion support for additional biosensors

- Reuse of existing open-source projects/firmware

## Deliverables

- Functional wearable prototype

- PCB + manufacturing files

- Firmware source code

- Mobile data protocol documentation

- Production-ready documentation

- Support during first PCBWay manufacturing run

## Ideal Candidate

Experience with:

- Wearable electronics

- Biomedical sensors

- BLE firmware

- Low-power embedded systems

- PCB design for compact devices

- Physiological signal acquisition

- Rapid prototyping for manufacturing

- 3D printing and design

Experience designing devices for:

- Research

- Healthtech

- Clinical monitoring

- Physiological sensing
 is highly preferred.

## Important Notes

The primary value of this device is clean raw data capture, not consumer smartwatch functionality. It is to be used in a research setting so multiday battery life etc are not required, however it should be relatively easy to setup and use.

Signal quality, manufacturability, low cost, and reliable streaming are more important than polished consumer features.

Experience working with PCBWay manufacturing constraints is highly desirable.