| 주간업무 회의 (Design Review) |  |  |  |  |  |  | Prepared by |  | Jibon |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Reviewed by |  |  |  |  |  |
|  |  |  |  |  |  |  | Approved by |  | 이 훈 규 |  |  |  |
| Department. R&D |  |  |  | Name : |  | Jibon |  |  |  |  |  |  |
| Title : Weekly Report |  |  |  |  |  |  |  |  |  |  |  |  |
| Meeting date : | 2026-Apr-19 | Working Duration |  |  | 2026-Mar-30 |  |  | 부터 |  | 2026-Apr-19 |  |  |
| 보고 내용 |  |  |  |  |  |  |  |  |  |  |  |  |

| Item | Do | Check / Plan | Resp. |
| --- | --- | --- | --- |
|  | Setting up Development Environment for qCardipia Application. There are 2 phase of development environment. - Yocto Build Development - Application Build & Debug Development Yocto Build Environment is setup done and already documented. qCardipia application build and debug environment setup require QtCreator IDE setup in some VM Linux system or Physical Linux system. The complete process is documented. The setup can be done by just following the documentation. |  |  |
|  | Bluetooth work around commands: To maintain and develop any feature for qCardipia it is required to work with QtCreator Plus Remote debugging setup. Since Veriscite Yocto Linux uses Waston or Wayland GUI, it is required to set some environment variables in QtCreator. The documentation contains complete list of env variables. Wifi and Bluetooth are integrated in Variscite Module. Wifi is Activated by default kernel setting from yocto. Qcardipia now using Native Linux Bluetooth features. |  |  |
|  | Commands Used by Qcardipia. hciconfig hci0 up hciconfig hci0 class 0x801F00 hciconfig hci0 down hciconfig hci0 up hciconfig hci0 piscan sdptool add --channel=22 SP echo Y > /sys/module/rfcomm/parameters/disable_cfc /* Default Linux settings hci0: Type: BR/EDR Bus: USB BD Address: 38:A2:8C:76:13:AD ACL MTU: 820:8 SCO MTU: 255:16 DOWN RX bytes:1225 acl:0 sco:0 events:122 errors:0 TX bytes:23230 acl:0 sco:0 commands:122 errors:0 */ return BTD_RTL8723BU; Bluetooth is activated in Variscite Linux and Qcardipia accessing Native Linux bluetooth. It is required to enable Linux Bluetooth in the QtCreator .pro project file. Note that the GIT repo contains 2 branches. Pull or change to AM62 Branch. Open the .pro project file name containing *AM62*. Edit .pro file and Insert LINUX_BLUETOOTH Follow documentation for full project setup to build and debug. |  |  |